Instrument Control Project

(TCP/IP based control and streaming)

Proof of concept document

Contents

1.	Introduction	3

2.	Demonstration objectives	4

2.1.	Requirements, list of features:	4

3.	Required Hardware:	5

4.	Required Software	6

4.2.	Delivered Software package	8

5.	Remote Control – Description of commands	8

5.1.	Introduction	8

5.2.	Notation	8

5.3.	Common Commands	8

*IDN?	8

5.4.	ABORt subsystem	8

5.5.	SENSe subsystem	8

SENSe:FREQuency Subsystem	8

SENSe:BANDwidth subsystem	9

SENSe:RLEVel subsystem	9

5.6.	DISPlay subsystem	9

VBW – Video Bandwidth	9

DISP:AVERage:MODE	10

DISP:AVERage:SAMPles	10

DISP:TRACe	10

5.7.	MEASurement subsystem	10

MEAS:MEASure	10

MEASurement:CONTinuous {0,1}	10

MEASurement:ABORt	10

5.8.	Other subsystems	10

6.	Remote Control – Examples	11

6.1.	Python example (test_tcp.py)	11

6.2.	SCPI scripting example (pxi-01.txt, pxi-02.txt, pxi-03.txt):	11

7.	Streaming channel	12

7.1.	Data format	12

7.3.	Example	13

Sending 10k samples, all zeros	13

F0=1, df=1, then 11000 pcs of “-2”:	14

8.	Demo	15

9.	ToDo list	15

| Version | Date | Author |
| --- | --- | --- |
| Ver1 | May 13 2016 | Peter Vago |
| Ver3 (delivered) | June 25 2016 | Peter Vago |
| Ver4 | June 27 2016 | Peter Vago |
| Ver5 (netcat client example added | Aug 22 2016 | Peter Vago |

# Introduction

This document was created to give a quick overview of designed features and implementation, timeline of this Proof of Concept (PoC) project.

PoC implementation and deliverables can be distributed under the NI sample code license. (http://ni.com/samplecodelicense )

PoC deliverables are:

- Documentation

- Reference design*

- Demonstration (remotely by Systems Engineer)

*Reference design/ ref. application:

Open source code shared with the NI platform community (e.g. published on ni.com, LV Tools Network or a public VIPM repository), intended to

-Provide examples of how to use specific features, or develop parts or the whole of an application

-Provide reference code that a developer may use as a template or higher-level starting point for developing all or part of a new application. Reference code is intended to be copied and modified for specific applications.

-Promote and teach best practices in software design and architecture

This code is distributed under the Sample Code License.

A reference application may be distributed as a Sample Project so that developers can easily create a new instance of the application.

# Demonstration objectives

The Demonstration Application intends to demonstrate how NI PXI Instruments can be controlled by clients over TCP/IP and what the streaming capabilities of NI PXI systems are over network.

PoC will deliver a single connection, single device control channel.

Device will be an NI VSA (Vector Signal analyzer), e.g. NI 5663 or 5665.

Figure 1: Single link, single device (picture from VXI-11 std)

Protocol will be designed according to VXI-11 standard, probably with some similarities to SCPI-99 protocol.

1. ábra: Connections, ports

## Requirements, list of features:

Instrument server control capabilities:

- Instrument (VSA)

  - Open session

  - Close session

  - Set/Get Frequency (center freq.)

  - Set/Get Span or Set/Get Start and Stop freq.

  - Set/Get RBW (Resolution bandwidth)

  - Run/Stop Acquisition

  - Run Measurements

    - Power level of highest peak in the Span

    - Peak frequency

  - Get Error and Status messages

- Streaming

  - Set server port (PXI will be the server/listener)

  - Set IP address (it will be fixed for first..)

  - Initiate session

  - Release session

# Required Hardware:

- PXIe chassis  + Controller

- 1xNI VSA** (NI 566x) -> Important: Name it as “NI_VSA” in MAX. This is hardcoded in the server software. (later improvement ->this can be read from INI file.)

- (Optional: NI 5690 or 5691 Preamplifier)

- RF signal source or antenna

Figure 2: Demo System (PXIe with VSA and VSG)

**Supported VSA types (all NI-RFSA based devices):

# Required Software

- Server:

  - LabVIEW 2014 Runtime-Engine (for executable)

  - LabVIEW 2014 (for source code)

    - A few other LabVIEW libraries are needed (OpenG) and others. I’ll try packing everything together in a single project…

Figure 3: Front Panel of server application

Figure 4: LabVIEW project of Server application

- Client:

  - LabVIEW 2014 for LabVIEW TCP Streamer client example (ex_TCP_Client.vi), or LV RunTime-Engine for executable

  - Python 3.x for Python client example (+pyvisa library, +NI-VISA driver OR pyvisa-py backend installed on client machine)

Figure 5: Python client example (snippet)

## Delivered Software package

# Remote Control – Description of commands

Control port:  40010

## Introduction

## Notation

## Common Commands

### *IDN?

Not implemented yet.

## ABORt subsystem

Not implemented yet.

## SENSe subsystem

### SENSe:FREQuency Subsystem

SENSe:FREQuency:CENTer 0 to fmax

This command defines the center frequency of the VSA or the measuring frequency for span = 0.

Example:

"FREQ:CENT 100MHz"

Characteristics:

*RST value: fmax/2 with fmax= maximum frequency

SCPI: conform

SENSe:FREQuency:SPAN 0 to fmax

In analyzer mode, this command defines the frequency span.

Example:

"FREQ:SPAN 10MHz"

Characteristics:

*RST value: fmax with fmax = maximum frequency

SCPI: conform

### SENSe:BANDwidth subsystem

SENSe:BANDwidth|BWIDth[:RESolution] 10 Hz to max.

This command defines the resolution bandwidth.

Analog resolution filters of 10 Hz to 20 MHz in 1, 2, 3, 5, 10 steps are available. Additionally there is a

50 MHz resolution bandwidth. These filters are implemented as 5-circuit LC filters in the range from

300 kHz to 10 MHz and as digital filters with analog characteristic in the range of 10 Hz to 100 kHz.

Example: "BAND 1MHz"

’Sets the resolution bandwidth to 1 MHz

Characteristics: *RST value: - (AUTO is set to ON)

SCPI: conform

### SENSe:RLEVel subsystem

SENSe:RLEVel -100 to 10

This command defines the Reference Level in dBm

## DISPlay subsystem

### VBW – Video Bandwidth

VBW is emulated in Vector Signal Analyzers: http://www.ni.com/tutorial/13869/en/

This example is used for implementing it: https://decibel.ni.com/content/docs/DOC-22813

(VBW Example.vi

When trying to measure signals in the presence of noise, the Video Bandwidth (VBW) filter can be used to reduce the variation in the noise while leaving the signal of interest unaffected. In order to emulate a VBW filter on hardware that does not have a physical filter, VBW can be applied in software using averaging dictated by the following equation:

N = [(k*RBW/VBW)^p + 1]^(1/p)

Where

k: .536

p: 1.275

N: Number of Averages

)

DISPlay:VBW 0 to max

Values: 0 ->bypass, >0 -> values in Hz

### DISP:AVERage:MODE

DISPlay:AVERage:MODE {OFF,RMS,PEAK}

Off: Disabled

RMS: RMS

PEAK: Peak Hold

### DISP:AVERage:SAMPles

Number of averages. Minimum 1, maximum 100 (not limited on server, but not tested with >100)

DISPlay:AVERage:SAMPles 1 to max

DISP:TRACe 1

Used for Enable trace on Soft Front Panel. Disabling trace can lead to higher performance.

Enabled: 1 , Disable: 0 (Default: 0)

DISPlay:TRACe {0,1}

## MEASurement subsystem

### MEAS:MEASure

Taking measurement (and streaming data through Streaming channel)

Measurement can be single-shot or continuous. For switching this option, see MEAS:CONT command.

MEAS:MEASure

(MEAS:STARt)

### MEASurement:CONTinuous {0,1}

Used for enabling Continuous measurement.

If this option is disabled, then instrument will take only one measurement after sending MEAS:MEAS command.

Enable: 1, Disable: 0 (Default 0)

MEAS:CONTinuous {0,1}

### MEASurement:ABORt

This is used only in continuous mode. After sending MEAS:MEAS the instrument will start measuring continuously. Measurement can be stopped by sending this MEAS:ABORt command.

MEAS:ABORt

(MEAS:STARt)

## Other subsystems

# Remote Control – Examples

## (Python install)

### Windows:

- Install python: from Npack package manager of from the official webpage

  - Add environment variables, as written here: http://stackoverflow.com/questions/3701646/how-to-add-to-the-pythonpath-in-windows-7

    - Create new variable Name: “PY_HOME”, Variable Value: “C:\path\to\python\version”

    - Add the following to the existing „Path” variable: „%PY_HOME%;%PY_HOME%\Lib;%PY_HOME%\DLLs;%PY_HOME%\Lib\lib-tk; „

    - Then you can enter “python” to command line and it will start python shell (type exit() to exit)

- Install Python libraries with pip (Pyhton Installer Package)

  - Run cmd as administrator

  - Install pip :  “python get-pip.py” (get-pip.py is attached to delivered zip package)

  - Install VISA library. You have 2 options:

    - 1) Install py-visa: “python -m pip install pyvisa” (this will download packages from the Internet) . This is based on NI VISA driver (supported for Windows and for some linux distros)

    - 2) Install pyvisa-py: “python -m pip install pyvisa” (this will download packages from the Internet)

## Python example (test_tcp.py)

Server has to be running first.

Syntax for sending SCPI commands:

python test_tcp.py scpi <string [parameter]>

Syntax for running scripts:

python test_tcp.py file <file_path>

## SCPI scripting example (pxi-01.txt, pxi-02.txt, pxi-03.txt):

# Test - Peter Vago, NI

# This is a multi-sequence script, 2398-2404MHz

#----------

#Settings, Trace enable, set single measurements

DISP:TRAC 1

MEAS:CONT 0

#------------

SENS:FREQ:SPAN 30M

SENS:BAND:RES 100k

SENS:RLEV -5

#----------

#Spectrum Config

SENS:FREQ:CENT 2398M

#----------

#Measure once

MEAS:MEAS

#----------

#Spectrum Config

SENS:FREQ:CENT 2399M

#----------

#Measure once

MEAS:MEAS

#----------

#Spectrum Config

SENS:FREQ:CENT 2400M

#----------

## 6.3 Putty example

I do not recommend using Putty for testing. Reason: http://the.earth.li/~sgtatham/putty/0.61/htmldoc/Chapter3.html#using-rawprot

## 6.4. netcat

netcat is a default tool on most Linux distributions. Can be installedon Windows too : http://kudithipudi.org/2011/07/14/how-to-use-netcat-nc-on-windows-7/

Example:

Single command:

echo "SENS:FREQ:CENT 2450M" | nc <server ip> <server port>

Multiple commands:

1. write your commands into a text file (my_commands.txt)

2. cat my_commands.txt | nc <server ip> <server port>

# Streaming channel

Port:40011

## Data format

## Example

### Sending 10k samples, all zeros

### F0=1, df=1, then 11000 pcs of “-2”:

References:

- https://zone.ni.com/reference/en-XX/help/371361J-01/lvconcepts/how_labview_stores_data_in_memory/

- https://en.wikipedia.org/wiki/Double-precision_floating-point_format

# Demo

First video demo is uploaded to Youtube.com : https://youtu.be/vN_r_bdAizM

Figure 6: Setup that used for video demo

# ToDo list

- Read initial values form INI file

- Design: Better queue organization (classes)

- Finishing: TCP streaming of Spectrum data

- Testing/finishing queries (“*IDN?”, all other parameters that is defined in this document)

- Compiling and testing server executable

- Performance measurement (pxi-03.txt script -> 8 steps)

698ms (Trace disabled) and 810ms (Trace enabled)

- Memory leak detected: Queue references were not closed properly. Fixed (June 27)

Document created in June 2016.

Peter Vago – Systems Engineer (peter.vago@ni.com,  + 36 30 493 4016)
National Instruments Europe